Firewall configuration

ciscoasa#configure terminal

ciscoasa(config)#

ciscoasa(config)#no

ciscoasa#

%SYS-5-CONFIG_I: Configured from console by console

ciscoasa#configure t

ciscoasa#configure terminal

ciscoasa(config)# no dhcpd address 192.168.1.5-192.168.1.36 inside

ciscoasa(config)#do show running-config

ciscoasa#configure t

ciscoasa(config)#int vlan 1

ciscoasa(config-if)#ip address 10.1.1.1 255.0.0.0

ciscoasa(config-if)#nameif inside

ciscoasa(config-if)#security-level 100

ciscoasa(config-if)#int et0/0

ciscoasa(config-if)#switchport access vlan 1

ciscoasa(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan2, changed state to down

ciscoasa(config-if)#exit

ciscoasa(config)#int VLAN 2

ciscoasa(config-if)#ip address 50.1.1.2 255.0.0.0

ciscoasa(config-if)#no shutdown

ciscoasa(config-if)#nameif outside

ciscoasa(config-if)#security-level 0

ciscoasa(config-if)# int et0/1

ciscoasa(config-if)#switchport access vlan 2

ciscoasa(config)#dhcpd address 10.1.1.10-10.1.1.30 inside

ciscoasa(config)#dhcpd dns 8.8.8.8 INterface inside

ciscoasa(config)#route outside 0.0.0.0 0.0.0.0 50.1.1.1

ciscoasa(config)#object network ?

configure mode commands/options:

WORD Specifies object ID (1-64 characters)

ciscoasa(config)#object network LAN

ciscoasa(config-network-object)#sub

ciscoasa(config-network-object)#subnet 10.0.0.0 255.0.0.0

ciscoasa(config-network-object)# nat (inside,outside) dynamic interface

ciscoasa(config)#exit

ciscoasa#write memory

Building configuration...

Cryptochecksum: 548844e6 1e0257a6 5e241f83 0df32841

902 bytes copied in 2.556 secs (352 bytes/sec)

[OK]

ciscoasa#configure terminal

ciscoasa(config)#access-list ofi extended permit tcp any any

ciscoasa(config)#access-list ofi extended permit icmp any any

ciscoasa(config)#access-group ofi in interface outside

ciscoasa(config)#exit

Router side (code)

Router>

Router>en

Router#configure t

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#int fa0/0

Router(config-if)#ip address 50.1.1.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

Router(config-if)#exit

Router(config)#int

Router(config)#interface fa0/1

Router(config-if)#ip address 8.8.8.1 255.0.0.0

Router(config-if)# no sh

Router(config-if)# no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/1, changed state to up

Router(config-if)#

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#router ospf ?

<1-65535> Process ID

Router(config)#router ospf 1

Router(config-router)#network 50.0.0.0 0.255.255.255 area 0

Router(config-router)#network 8.0.0.0 0.255.255.255 area 0

Router(config-router)# do wr

Building configuration...

[OK]

Router(config-router)#